# Manual Roll Packing Ergonomic Feasibility Study

## 1. Purpose

The purpose of this study is to determine the feasibility of using two-person manual handling to lift, move and pack cylindrical rolls of material in an industrial production environment.

Two products are considered:

| Parameter | 15-ft product | 25-ft product |
| --- | --- | --- |
| Material length | 15 ft wound around core | 25 ft wound around core |
| Finished roll width | ~48 in | ~48 in |
| Roll OD | 12–15 in | 12–15 in |
| Core diameter | 3 in | 5 in |
| Roll weight | TBD | 145 lb / 65.8 kg |
| Workers | 2 | 2 |
| Approx. handling height | 36 in | 36 in |
| Production target | 85–100/shift | 65/shift |
| Shift duration | 12 h including breaks/setup | 12 h including breaks/setup |

The assessment considers:

NIOSH Revised Lifting Equation (RNLE)
 Liberty Mutual/Snook psychophysical methodology
 Team-lifting principles
 Biomechanics and ergonomics
 Ontario occupational health and safety requirements

# 2. Preliminary conclusion

Based on currently available information, I would classify:

25-ft / 145-lb product:

Two-person manual packing is not yet demonstrated to be ergonomically acceptable and should not be approved solely on the basis of two workers being capable of lifting 145 lb.

The nominal load division is:

145/2=72.5 lb/person145/2=72.5\text{ lb/person}

but ergonomic analysis cannot assume each employee experiences exactly 72.5 lb. Team strength is less than the sum of individual strength because of coordination, unequal loading, differences in worker height, grip and timing. CCOHS specifically warns of this limitation with team handling.

For the 15-ft product, a conclusion cannot yet be made because its weight is unknown.

The 36-in working height is favorable. The major concerns are instead:

load magnitude + reach + coupling + lowering/placement + team coordination + cumulative 12-hour exposure.

# 3. Legal framework — Ontario

There is no simple Ontario law stating that an employee may lift no more than X pounds.

The applicable requirement is broader.

Ontario Regulation 851, s.45 requires materials that are lifted, carried or moved to be handled with precautions and safeguards so that the activity does not endanger workers. It also requires materials to be placed/stored so they will not tip, collapse or fall and can subsequently be removed safely.

This is especially relevant because your products are cylindrical rolls. Section 47 additionally requires cylindrical objects stored on their sides to be appropriately arranged and chocked/wedged to prevent movement.

Ontario's ergonomics guidance recognizes both cumulative exposure from repeated handling and one-time high-force exertions as potential causes of MSDs. It identifies engineering controls, mechanical assistance and task redesign as controls, while team lifting, job rotation and training are administrative measures. Ontario specifically cautions that job rotation and training are not effective controls when used alone.

Therefore, saying:

"There are two workers and both can lift it."

would not be an adequate ergonomic justification.

# 4. Understanding the NIOSH theory

The Revised NIOSH Lifting Equation is one of the most established tools for evaluating manual lifting.

It starts from an ideal Load Constant of 51 lb (23 kg).

NIOSH then reduces that ideal load according to the actual conditions of the job.

The equation is:

RWL=LC×HM×VM×DM×AM×FM×CM\boxed{RWL=LC\times HM\times VM\times DM\times AM\times FM\times CM}

where:

| Variable | Meaning |
| --- | --- |
| LC | Load Constant = 51 lb |
| HM | Horizontal Multiplier |
| VM | Vertical Multiplier |
| DM | Distance Multiplier |
| AM | Asymmetric Multiplier |
| FM | Frequency Multiplier |
| CM | Coupling Multiplier |

The result is the:

### Recommended Weight Limit — RWL

Then:

LI=Actual LoadRWL\boxed{LI=\frac{Actual\ Load}{RWL}}

which produces the Lifting Index.

NIOSH describes RNLE as an assessment method for two-handed manual lifting, and its application has defined limitations.

# 5. How to interpret Lifting Index

Conceptually:

LI≤1LI\leq1

means the load does not exceed the calculated Recommended Weight Limit under the evaluated conditions.

If:

LI=2LI=2

the actual load is twice the calculated RWL.

If:

LI=3LI=3

the load is three times the calculated RWL.

So don't interpret LI as:

"There is a 3× chance of injury."

It does not provide an injury probability.

Instead, higher LI indicates increasing physical demand and increasing concern relative to the design criterion.

# 6. Why the 51-lb number isn't a legal lifting limit

This distinction will be important when you present the study.

NIOSH does not mean:

"Nobody may lift more than 51 lb."

The 51-lb load constant represents ideal lifting circumstances.

Real workplaces are rarely ideal.

Imagine someone lifting a compact object with:

- good handles

- two hands

- object close to body

- favorable height

- no twisting

- low frequency

- short vertical displacement.

That approaches the ideal condition.

Your roll isn't necessarily like that.

Its 12–15-in diameter and 48-in width may make the worker reach around or away from their body.

That reduces the allowable load.

# 7. Horizontal reach — probably one of your biggest factors

Biomechanically, the farther the load is from the spine, the greater the bending moment.

Very simply:

Moment=Force×DistanceMoment=Force\times Distance

Suppose a worker supports 72.5 lb.

At 10 inches from the body:

M=72.5(10)=725 lb-inM=72.5(10)=725\text{ lb-in}

At 20 inches:

M=72.5(20)=1450 lb-inM=72.5(20)=1450\text{ lb-in}

The load hasn't changed.

But the external moment has approximately doubled.

This is why:

Keeping the load close to the body can be as important as reducing its weight.

Liberty Mutual similarly identifies reach as an important risk factor because increasing the distance between the load and body increases physical stress on the back, shoulders and arms.

# 8. NIOSH horizontal multiplier

NIOSH represents this approximately through:

HM=10HHM=\frac{10}{H}

where HH is the horizontal hand location in inches.

Consider an illustrative measurement of:

H=15"H=15"

Then:

HM=10/15HM=10/15 HM=0.667HM=0.667

Before considering anything else:

51(0.667)=34.0 lb51(0.667)=34.0\text{ lb}

So simply having the hands 15 inches horizontally away significantly reduces the recommended load compared with ideal conditions.

This is why I want an actual measurement from the station rather than estimating it.

# 9. Vertical height

You estimated approximately:

V=36"V=36"

This is comparatively favorable because the material is being handled around waist/knuckle height rather than from the floor or above the shoulders.

CCOHS identifies the preferred lifting region generally as above the knees/mid-thigh and below the shoulders/mid-chest.

Therefore:

### Working height = relatively good

and isn't currently the main ergonomic problem.

But we still need the destination height.

Picking something up at 36 inches and placing it at 36 inches is completely different from:

36" → carry → lower to 10"

or:

36" → lift → stack at 55".

# 10. Asymmetry

Another important variable is rotation.

If the operators pick the roll up like this:

machine → lift → walk straight → place straight

that's much better than:

machine → lift → rotate body 90° → walk → turn → lower.

Twisting under load increases biomechanical demand.

Liberty Mutual's current tool specifically notes that twisting is not separately included in its equations. Therefore, a good Snook/Liberty result doesn't automatically mean a task involving significant twisting is acceptable.

That's another reason we need observational assessment in addition to a numerical calculation.

# 11. Coupling/grip

Your rolls have:

15-ft product → 3-in core

25-ft product → 5-in core

but core diameter alone doesn't establish good coupling.

We need to see where the workers put their hands.

For example:

### Situation A — dedicated handles

Potentially good coupling.

### Situation B — hands inside core

Potentially reasonable depending on wrist posture and access.

### Situation C — grabbing material around 12–15" OD

Potentially fair/poor coupling.

### Situation D — fingertips under roll

Poor and potentially hazardous.

CCOHS recommends adequate grip points as part of reducing lifting risk.

For this particular product, I would pay close attention to wrist posture and finger/pinch hazards when the roll is lowered.

# 12. Snook / Liberty Mutual theory

The Snook tables approach the problem differently from NIOSH.

NIOSH is largely asking:

"What recommended load results from these lifting conditions?"

The Snook/Liberty Mutual approach is psychophysical.

Workers in research studies performed material-handling tasks and adjusted or evaluated loads based on what they believed they could perform repeatedly without becoming unusually fatigued or overexerted.

The modern Liberty Mutual equations provide a:

Population Percentile\boxed{\text{Population Percentile}}

This estimates the percentage of a population expected to perform a specified task without perceived overexertion.

# 13. How to interpret a Snook result

Suppose our eventual calculation produces:

### 90% female

That means approximately 90% of the reference female working population would be expected to perform the specified task without perceived overexertion.

Suppose it gives:

### 75%

That's around an important design boundary.

Suppose it gives:

### 30%

Then only about 30% of the reference population is expected to find that task acceptable without overexertion.

The interpretation is therefore:

Lower population percentage=greater concern\boxed{\text{Lower population percentage}=\text{greater concern}}

Liberty Mutual currently states that jobs should ideally be designed for at least 90% of the female population, and identifies tasks below 75% female capability as exposing workers to substantially higher injury risk. It also explains that the female percentile is useful for protecting both male and female workers, rather than being a statement that women alone are at risk.

That distinction is useful when presenting this study to management.

# 14. Why we shouldn't simply put 72.5 lb into Snook

This is extremely important.

The load is:

145 lb145\text{ lb}

There are two workers.

It is tempting to say:

145/2=72.5 lb145/2=72.5\text{ lb}

and run Snook for a 72.5-lb lift.

That assumes:

F1=F2=72.5F_1=F_2=72.5

throughout the entire operation.

That's unlikely to remain true.

Imagine one worker begins lifting 0.3 seconds before the other.

For that brief period:

F1>72.5F_1>72.5

Similarly, when:

- one worker is taller;

- one end is lowered first;

- one worker walks backward;

- one person turns;

- one grip slips;

- the roll isn't perfectly centred;

- workers accelerate/decelerate differently,

the load distribution changes.

CCOHS specifically states:

the combined strength of a team is less than the sum of the individual strengths.

It recommends similar-height/strength team members, a designated leader, predetermined commands and practice.

Therefore, 72.5 lb/person is a nominal static estimate—not proof of actual individual exposure.

# 15. Production frequency

For the 25-ft product:

65/12=5.42 rolls/hour65/12=5.42\text{ rolls/hour}

Average interval:

60/5.42=11.1 min/roll60/5.42=\mathbf{11.1\ min/roll}

For the 15-ft product:

85/12=7.08/hr85/12=7.08/hr

to

100/12=8.33/hr100/12=8.33/hr

or approximately one roll every:

7.2−8.5 minutes\mathbf{7.2-8.5\ minutes}

But these numbers shouldn't go directly into the ergonomic model.

Why?

Because the 12 hours includes:

- breaks

- setup

- changeovers

- downtime

- cleaning

- other duties.

If 65 rolls are actually packed during 7 production hours:

65/7=9.3/hr65/7=9.3/hr

That's substantially different.

Therefore the final study needs:

Actual lifts/min during active production\boxed{\text{Actual lifts/min during active production}}

rather than simply shift average.

# 16. Cumulative workload

For the 25-ft product:

145×65=9,425 lb145\times65=\mathbf{9,425\ lb}

The team therefore handles approximately:

4,275 kg\mathbf{4,275\ kg}

of material per shift.

Nominally:

9,425/2=4,712.5 lb/worker9,425/2=4,712.5\text{ lb/worker}

or:

2,137 kg/worker\mathbf{2,137\ kg/worker}

per shift.

This doesn't mean each worker "lifts two tonnes at once", nor is cumulative tonnage an ergonomic limit.

It is simply a useful exposure indicator demonstrating why:

moderate frequency × high individual load × long shift

deserves attention.

# 17. 12-hour shift considerations

This is another reason I would not use one calculation as the sole decision criterion.

NIOSH's RNLE has defined conditions and limitations; its manual notes that application outside the assumptions can under- or overestimate physical stress.

A 12-hour production shift also introduces fatigue considerations that aren't adequately represented by simply dividing production by 12 hours.

The study should therefore combine:

quantitative assessment

with

task observation + fatigue/exposure assessment.

# 18. Acute safety risks

The feasibility study shouldn't focus only on MSDs.

A 145-lb cylindrical object introduces acute hazards as well.

### Dropped roll

Could injure:

- feet

- ankles

- hands

- fingers.

### Rolling

A 12–15-in diameter object naturally has potential to roll.

### Pinch points

Particularly during final placement.

### Finger trapping

One worker may lower before the other.

### Loss of synchronization

Especially if operators can't see each other clearly.

### Slips/trips

Particularly dangerous while carrying a 145-lb shared load.

### Unexpected production issue

Material deformation, damaged cores or loose wrapping could alter grip.

Ontario Regulation 851 requires material handling not to endanger workers and specifically addresses stability of cylindrical materials.

# 19. Preliminary risk assessment

Using a qualitative engineering assessment:

| Hazard | 15-ft | 25-ft |
| --- | --- | --- |
| High load | TBD | High |
| Awkward reach | Measure | Potentially high |
| Poor grip | Measure | Potential |
| Repetition | Moderate | Moderate |
| 12-h exposure | Significant | Significant |
| Team coordination | Significant | Significant |
| Back loading | TBD | High concern |
| Shoulder loading | TBD | High concern |
| Hand/finger pinch | Significant | Significant |
| Dropped load | TBD | Significant |
| Rolling load | Significant | Significant |
| Twisting | Observe | Observe |
| Overall | Pending weight | Further controls/analysis required |

# 20. Feasibility of current concept

I would currently describe the 25-ft operation as:

### 🟠 Conditionally feasible only after further ergonomic assessment/control

rather than either:

"safe"

or

"impossible."

There isn't enough evidence to approve unrestricted two-person manual handling.

The 145-lb total weight remains substantial even with two operators.

# 21. Engineering-control hierarchy

The first question shouldn't be:

"How do we train employees to lift 145 lb?"

Instead:

"How can we reduce or eliminate the need to lift 145 lb?"

Ontario's guidance emphasizes engineering controls, while training/job rotation alone shouldn't be relied upon as the sole MSD controls.

I'd investigate the options in approximately this order:

A. Roll instead of lift

If the product can transfer from one cradle/roller surface directly into packaging, the workers may only need to guide it.

B. Roller conveyor

The roll remains supported while moving.

C. Height-matched packing table

Machine discharge:

36"36"

Packing surface:

≈36"\approx36"

Then eliminate vertical lifting.

D. Roll cradle/cart

Move the entire roll while supported.

E. Lift-assist device

Potential options include:

- articulated manipulator

- vacuum lifter if material permits

- hoist with roll attachment

- roll lifter

- powered lift table.

CCOHS recommends considering mechanical aids and lifting equipment when manual handling can't otherwise be adequately reduced.

# 22. A potentially inexpensive solution

You may not need an expensive robot.

If the machine produces the roll around:

36"36"

and packaging can be brought to approximately the same height, I'd investigate a:

### Cradle + roller transfer system

Conceptually:

Machine

→ roll rests in cradle

→ operator releases stop

→ roll moves across rollers

→ packing cradle

→ wrap/secure

The employees would primarily:

guide + rotate + position

rather than:

lift + carry + lower.

That fundamentally changes the ergonomic problem.

Instead of evaluating a 145-lb lift, we'd potentially evaluate a push/pull force.

That can be measured with a force gauge and evaluated using Liberty Mutual's push/pull methodology.

Often that's a much better engineering solution.

# 23. What would make the study complete

We currently know perhaps 70% of what we need.

Collect these measurements during an actual production run:

| Measurement | Needed |
| --- | --- |
| 15-ft roll weight | Yes |
| 25-ft weight | 145 lb ✓ |
| Roll width | ~48" ✓ |
| OD | 12–15" ✓ |
| Starting hand height | ~36" ✓/verify |
| Ending hand height | Measure |
| Horizontal hand distance | Measure |
| Vertical travel | Measure |
| Carrying distance | Measure |
| Rotation angle | Measure |
| Actual active-production lifts/min | Measure |
| Grip location | Observe |
| One worker walking backward? | Observe |
| Actual packing time/roll | Measure |
| Rest time between handling cycles | Measure |
| Floor condition | Observe |
| Worker posture | Observe |
| Final roll placement | Observe |

Video recording a few cycles from the side and front—subject to your workplace's rules—would make the assessment considerably better.

# 24. Final acceptance framework

I would use multiple criteria rather than one magic number.

### NIOSH

Calculate:

RWLRWL

and:

LI=LoadRWLLI=\frac{Load}{RWL}

Use it as a lifting-risk indicator while explicitly documenting its applicability limitations to team handling.

### Snook/Liberty Mutual

Calculate population capability where the task fits the method.

Target:

≥90% female\boxed{\geq90\%\ female}

with:

<75% female\boxed{<75\%\ female}

treated as an important warning threshold under Liberty Mutual's interpretation.

### Biomechanics

Assess:

- reach

- trunk flexion

- rotation

- shoulder position

- wrist position

- grip

- balance.

### Team handling

Assess:

- synchronization

- worker positioning

- load sharing

- communication

- visibility

- walking direction.

### Acute safety

Assess:

- dropping

- rolling

- pinch/crush

- slips/trips

- packaging instability.

### Ontario OHSA/Reg. 851

Demonstrate that reasonable precautions and safeguards are being used so material movement doesn't endanger workers.

# Overall preliminary feasibility conclusion

15-ft roll

Assessment pending. The weight must be measured before determining feasibility. The higher production target of 85–100 rolls per shift means frequency/exposure may become more important even if individual roll weight is substantially lower.

25-ft / 145-lb roll

Current two-person manual handling should not yet be considered ergonomically validated. Although two workers reduce the nominal individual load to approximately 72.5 lb, this assumes perfect load sharing and does not account for reach, grip, posture, asymmetric loading, placement height, team coordination or fatigue. Team-handling guidance also establishes that combined team capability cannot simply be treated as the arithmetic sum of individual capabilities.

The 36-in handling height is favorable, and the relatively low nominal cycle frequency helps. However, the high individual load magnitude, 12-hour shift, cylindrical geometry, potential reach and team-handling variability remain significant concerns.

The next engineering step should therefore be to measure the missing task variables and evaluate whether the physical lift can be eliminated or substantially reduced through a height-matched roller/cradle/lift-assist arrangement. CCOHS similarly recommends reducing load demands and using suitable lifting equipment where practical.

Once you give me the 15-ft roll weight and preferably a photo/sketch of the station showing where the roll starts and where the two workers put it, I can take this one step further and calculate the actual Snook/Liberty Mutual values and an RNLE screening calculation, then turn the whole thing into a professional feasibility-study report with Executive Summary, Scope, Methodology, Calculations, Ontario regulatory section, risk matrix, findings, proposed equipment concepts and final recommendation suitable for presenting to your Operations Director or Health & Safety team.